Study Guide

Real-Time Traffic Light Control System

Multi-Process IPC under Linux (System V)

ENCS4330 — Real-Time Applications & Embedded Systems

Birzeit University — Faculty of Engineering and Technology

Project #2 — 2nd Semester 2025/2026

A line-by-line walkthrough of every source file,

the IPC objects they share, and the runtime choreography

that turns nine independent programs into one safe intersection.

# How to Use This Guide

This guide is organized to be read in order, top to bottom, the same way the program is built. Each chapter takes one source file and explains what it does, why it does it that way, and which other files it talks to. If you only have an hour before the discussion, read Chapters 1, 2, 3, and 5 — those cover the global picture, the shared data, and the brain of the system.

### Conventions

- Code is shown in a gray monospace box. Line numbers in parentheses refer to the actual file.

- Yellow side-bar boxes hold extra explanations: "Why:" for design rationale, "Tip:" for practical advice, "Gotcha:" for things that bite you in C.

- "Blackboard" is our shorthand for the shared-memory region. "Mailboxes" means the three System V message queues.

- Directions are numbered 0=N, 1=S, 2=E, 3=W everywhere in the code, so an int can index any per-direction array.

### Source tree at a glance

traffic_system/

Makefile              # build rules for every binary

README.md             # quick-start and troubleshooting

config/

system.cfg          # all tunable timings and rates

src/

common.h            # shared types, enums, helpers (everyone includes this)

ipc_init.h          # System V IPC wrappers (ftok, shmget, semget, msgget)

config.h            # plain-text config parser

main.c              # launcher: creates IPC, forks 9 children, supervises

controller.c        # the brain: phase state machine, safety, adaptive timing

traffic_light.c     # one instance per direction; executes commands

vehicle_detector.c  # one per direction; spawns random arrivals

pedestrian.c        # random crossing button presses

emergency.c         # rare emergency-vehicle events

logger.c            # drains the LOG queue to logs/system.log

gui_opengl.c        # freeglut visualizer (reads shared memory only)

logs/

system.log          # timestamped event stream

docs/

design_report.pdf

study_guide.docx    # this file

# Chapter 1 — The Big Picture

Before we open any source file, you need a mental model of what runs and how it talks. The system is nine independent Linux processes that share three System V IPC objects. They were built as separate programs (not threads) for three reasons:

- Fault isolation. If one process segfaults, the others keep running and the supervisor can restart it.

- True preemption. The Linux scheduler treats each component as a first-class citizen, so a busy controller cannot starve the logger.

- Honesty about the assignment. The spec asks for IPC; if everything were threads in one process we would only need a mutex.

## 1.1 The nine processes

| Binary | Role | How many |
| --- | --- | --- |
| main | Launcher / supervisor / reaper | 1 |
| controller | Phase state machine; emits commands | 1 |
| traffic_light | Executes commands for one direction | 4 (N, S, E, W) |
| vehicle_detector | Random arrivals; sends events | 1 (spawns for all dirs) |
| pedestrian | Random crossing-button presses | 1 |
| emergency | Rare emergency-vehicle events | 1 |
| logger | Writes time-stamped log file | 1 |
| gui_opengl | freeglut 3-D visualization | 1 |

Why: traffic_light is replicated four times so each direction has its own command queue slot via the message-queue mtype field. The other components do not need replication; they each handle all four directions internally.

## 1.2 The three shared objects

| IPC object | Purpose | Who reads | Who writes |
| --- | --- | --- | --- |
| Shared memory (shm) | The blackboard — global state | All 9 | controller + traffic_light |
| Semaphore (sem) | Mutex protecting the blackboard | All 9 | All 9 |
| CMD message queue | controller → lights, lights → controller acks | controller + 4 lights | controller + 4 lights |
| EVT message queue | Sensors → controller | controller | vehicle/ped/emergency |
| LOG message queue | Anyone → logger | logger | everyone |

That is the entire IPC surface: one shm, one sem, three msq. Every other interaction is just a Linux signal (SIGTERM at shutdown).

## 1.3 The phase state machine, in five words

NS_GREEN → NS_YELLOW → ALL_RED → EW_GREEN → EW_YELLOW → ALL_RED → (repeat)

Two side branches splice into this loop:

- PEDESTRIAN phase — entered only from ALL_RED, runs once the controller sees a pending request.

- EMERGENCY phase — entered from any state by first yellowing then red-ing all conflicting directions, then granting green to the emergency direction.

## 1.4 The lifetime of one tick

Roughly twenty times per second the controller does this:

1.  semop(SEM_LOCK)          // grab the blackboard mutex

2.  read state from shm     // current phase, queues, requests

3.  drain EVT queue (NOWAIT) // pick up any new arrivals/requests

4.  decide next phase        // adaptive timing + safety check

5.  if phase changed:

enqueue CMD messages   // one per light that must change

wait for ACK on CMD    // confirm each light obeyed

update shm.phase

6.  semop(SEM_UNLOCK)        // release the mutex

7.  log to LOG queue         // outside the lock; never blocks

8.  usleep(50_000)           // 50 ms cadence

Tip: Logging is done after releasing the semaphore. If we logged while holding the lock, a slow disk write would freeze the GUI and the lights.

# Chapter 2 — Makefile and Build System

The Makefile is the simplest interesting file in the project. It compiles eight binaries from src/*.c and links the GUI binary against freeglut. We use plain GNU Make — no Autotools, CMake, or Meson — because the project has a fixed shape and adding a build system would obscure what is happening.

## 2.1 The variables

CC      = gcc

CFLAGS  = -Wall -Wextra -O2 -std=gnu99 -Isrc

LDFLAGS =

GLLIBS  = -lglut -lGLU -lGL -lm

- -Wall -Wextra turn on every reasonable warning. We made the code compile clean with both flags on; if a warning ever reappears, you broke something.

- -O2 is the standard release optimization level. It enables loop optimizations and inlining without the binary-size cost of -O3.

- -std=gnu99 gives us C99 plus the GNU extensions (anonymous structs in unions, designated initializers, __attribute__).

- -Isrc puts src/ on the include path so every .c file can say #include "common.h".

- GLLIBS collects the libraries needed only by the GUI binary. freeglut depends on the GLU utility library and the core GL library; -lm is for sin/cos used in the camera rotation.

## 2.2 The targets

all: main logger controller traffic_light \

vehicle_detector pedestrian emergency gui_opengl

main: src/main.c src/common.h src/ipc_init.h src/config.h

$(CC) $(CFLAGS) -o $@ $<

# ... one rule per binary ...

gui_opengl: src/gui_opengl.c src/common.h src/ipc_init.h

$(CC) $(CFLAGS) -o $@ $< $(GLLIBS)

Why: We do not build object files (.o) separately because each binary is a single translation unit. Compiling straight to an executable is shorter, easier to debug, and rebuilds are still fast — the whole project compiles in under two seconds.

## 2.3 Convenience targets

- make debug — rebuilds with -g -O0 -DDEBUG. Use this when you want gdb to step through; -O2 reorders statements and confuses single-stepping.

- make run — convenience that runs ./main config/system.cfg with the GUI.

- make run-nogui — same but passes --no-gui. Useful when grading remotely over SSH without X forwarding.

- make clean — removes binaries.

- make ipcclean — calls ipcrm for every IPC object the program might have left behind if it crashed before cleanup. Run this after a hard kill.

Gotcha: If you Ctrl-C the program in the wrong place (between IPC creation and the cleanup handler), the kernel does NOT auto-remove the IPC objects — they survive until reboot or ipcrm. The ipcclean target is your friend here.

# Chapter 3 — The Config File

Birzeit's specification is explicit: no hard-coded values. Every timing constant lives in config/system.cfg, which is read at startup. You change a number, you restart — no recompile.

## 3.1 Format

Plain text. Blank lines and lines starting with # are ignored. Everything else must look like:

KEY = VALUE

KEYs are case-sensitive ASCII identifiers; VALUEs are integers (timing constants are seconds, rates are percentages with one exception).

## 3.2 The keys, grouped

### Phase timing

| Key | Default | Meaning |
| --- | --- | --- |
| T_GREEN_MIN | 8 s | Floor for any green phase, even with no traffic |
| T_GREEN_MAX | 15 s | Ceiling, even with very heavy traffic |
| T_YELLOW | 3 s | Transition green → red |
| T_ALL_RED | 2 s | Safety gap with everything red |
| T_PEDESTRIAN | 10 s | Length of walk phase |
| T_EMERGENCY_RESPONSE | 5 s | Max time to grant emergency green |

### Deadlines

| Key | Default | Meaning |
| --- | --- | --- |
| T_PEDESTRIAN_MAX_WAIT | 45 s | After this, force a walk phase |
| T_VEHICLE_MAX_WAIT | 60 s | Boosts the cross-direction's adaptive timer |

### Stochastic input rates

| Key | Default | Meaning |
| --- | --- | --- |
| VEHICLE_SPAWN_RATE | 25 | Percent chance per tick of a new arrival |
| PEDESTRIAN_SPAWN_RATE | 10 | Percent chance per tick of a button press |
| EMERGENCY_SPAWN_RATE | 5 | Per-mille rate (i.e. 5 per 1000 ticks) |

Why: Vehicle and pedestrian rates are out of 100, but emergency is out of 1000 — emergencies are rare events and resolving 1% is too coarse. We document this in the config comments so the grader does not get confused.

## 3.3 How it is parsed (config.h)

The parser is intentionally dumb. We do not pull in a YAML or TOML library because the C standard library already has everything we need: strtok and atoi. The whole parser is forty lines.

static void apply_kv(config_t *c, const char *k, const char *v) {

int n = atoi(v);

if      (!strcmp(k, "T_GREEN_MIN"))     c->t_green_min = n;

else if (!strcmp(k, "T_GREEN_MAX"))     c->t_green_max = n;

else if (!strcmp(k, "T_YELLOW"))        c->t_yellow    = n;

/* ... one branch per key ... */

else fprintf(stderr, "config: ignoring unknown key '%s'\n", k);

}

- Unknown keys produce a warning, not an error. This lets us add experimental keys without breaking existing config files.

- Missing keys silently keep their compiled-in defaults set by load_defaults(). This is by design: a stripped-down config still works.

- If you mistype a key, scroll up after startup — every typo prints a line to stderr.

Tip: To experiment with adaptive timing, lower T_GREEN_MIN to 4 and raise T_GREEN_MAX to 20. You will see the controller stretch the green when one side is busier.

# Chapter 4 — common.h: the shared vocabulary

Every other file includes common.h. It defines the data types that travel through shared memory and the message queues, plus a few wrapper functions that are short enough to live in a header. This is the most important file to understand thoroughly.

## 4.1 The enums

typedef enum { DIR_N=0, DIR_S=1, DIR_E=2, DIR_W=3, NUM_DIRS=4 } direction_t;

typedef enum { LIGHT_RED=0, LIGHT_YELLOW=1, LIGHT_GREEN=2 } light_color_t;

typedef enum {

PHASE_INIT,

PHASE_ALL_RED,

PHASE_NS_GREEN, PHASE_NS_YELLOW,

PHASE_EW_GREEN, PHASE_EW_YELLOW,

PHASE_PEDESTRIAN,

PHASE_EMERGENCY_NS, PHASE_EMERGENCY_EW,

PHASE_SHUTDOWN

} phase_t;

Notice that directions are 0..3 specifically so an int can index every per-direction array (lights[dir], queues[dir], ped_request[dir]). We never use strings as direction keys — that would be slow and fragile.

Why: We have two separate emergency phases (NS and EW) instead of one parameterized phase. This makes the state machine flat: the controller knows from the phase alone which lights should be green, without needing to also remember the emergency direction. It is duplication for clarity.

## 4.2 The blackboard

typedef struct {

/* lights */

light_color_t lights[NUM_DIRS];

time_t        light_change_t[NUM_DIRS];

int           light_faulty[NUM_DIRS];

/* phase */

phase_t   phase;

time_t    phase_start;

int       phase_duration;

/* traffic */

int       vehicle_queue[NUM_DIRS];

time_t    first_arrival[NUM_DIRS];

/* pedestrians */

int       ped_request[NUM_DIRS];

time_t    ped_request_t[NUM_DIRS];

/* emergency */

int       emergency_active;

direction_t emergency_dir;

time_t    emergency_start;

/* book-keeping */

int       shutdown;

long      tick;

/* a copy of the config so processes that fork after startup

still see the same values */

config_t  cfg;

} shared_state_t;

A few subtleties worth pointing out:

- light_change_t stores when each light last changed color. The GUI uses this to drive a brief brightening animation; the logger uses it to compute dwell times.

- first_arrival[dir] is the wall-clock time of the oldest car still queued in that direction. The controller compares this against T_VEHICLE_MAX_WAIT to boost the adaptive green.

- cfg — we copy the whole parsed config into the blackboard. That way the GUI can render numerical timing info without re-parsing the file, and any later-spawned process can read the same parameters.

Gotcha: Never put a pointer in shared memory. The same address is meaningless in another process. The struct only contains plain integers, time_ts, enums, and fixed-size arrays — all safe to share.

## 4.3 The four message types

/* controller -> light */

typedef struct {

long mtype;          /* MTYPE_CMD_BASE + direction */

direction_t dir;

light_color_t target;

time_t when;

} cmd_msg_t;

/* light -> controller */

typedef struct {

long mtype;          /* MTYPE_ACK */

direction_t dir;

light_color_t reached;

int ok;              /* 0 = fault */

} ack_msg_t;

/* sensor -> controller */

typedef struct {

long mtype;          /* MTYPE_EVT_* */

direction_t dir;

int kind;            /* EVT_VEHICLE, EVT_PEDESTRIAN, EVT_EMERGENCY */

time_t when;

} evt_msg_t;

/* anyone -> logger */

typedef struct {

long mtype;          /* always 1 */

int  severity;       /* 0=INFO, 1=WARN, 2=ERROR */

char text[160];

} log_msg_t;

Why: Every message has 'long mtype' first because System V msgsnd/msgrcv treats it as the routing key. We use disjoint mtype ranges so the receiver can ask for exactly the messages it wants.

### How mtype routing works

- Light N reads with msgrcv(cmd_q, &m, sizeof(m)-sizeof(long), MTYPE_CMD_BASE+0, 0). It only receives commands addressed to direction 0.

- Light S asks for MTYPE_CMD_BASE+1, light E for +2, light W for +3.

- Controller asks for MTYPE_ACK to wait for any light's reply.

- Controller asks for 0 on the EVT queue, which means "any message", to drain the queue.

Tip: This trick — using mtype as a routing key — replaces what a thread-based system would do with four separate condition variables. It is one of the most underrated features of System V message queues.

## 4.4 The helpers

static inline void sem_lock  (int s) { struct sembuf op = {0,-1,SEM_UNDO}; semop(s,&op,1); }

static inline void sem_unlock(int s) { struct sembuf op = {0,+1,SEM_UNDO}; semop(s,&op,1); }

static inline void send_log(int q, int severity, const char *fmt, ...) {

log_msg_t m = { .mtype = 1, .severity = severity };

va_list ap; va_start(ap, fmt);

vsnprintf(m.text, sizeof(m.text), fmt, ap);

va_end(ap);

msgsnd(q, &m, sizeof(m)-sizeof(long), IPC_NOWAIT);

}

SEM_UNDO is the critical flag. If a process holding the semaphore is killed by SIGKILL (or it segfaults), the kernel automatically reverses its semop operations. Without SEM_UNDO, a crash inside the critical section would freeze every other process forever.

IPC_NOWAIT on msgsnd means "if the queue is full, drop this message rather than block." Logging must never deadlock the rest of the system.

## 4.5 The safety invariant

static inline int state_is_safe(const shared_state_t *s) {

/* NS and EW must not both be non-red simultaneously */

int ns_active = (s->lights[DIR_N] != LIGHT_RED) ||

(s->lights[DIR_S] != LIGHT_RED);

int ew_active = (s->lights[DIR_E] != LIGHT_RED) ||

(s->lights[DIR_W] != LIGHT_RED);

return !(ns_active && ew_active);

}

This is the cheapest line of defense against the entire safety requirement of the spec. Any time the controller is about to commit a state change, it asks this function and refuses if the answer is false. The function is so short the compiler inlines it everywhere.

Why: We could write a much fancier invariant — checking yellow-before-red transitions, pedestrian conflicts, etc. — but the spec's only HARD safety rule is "no conflicting greens." A simple invariant is easier to audit, and the other rules are enforced by the structure of the state machine itself.

# Chapter 5 — controller.c: the brain

This is the longest source file and the heart of the system. Read this chapter slowly. Everything else exists to feed information into the controller or to act on its commands.

## 5.1 What it does, in plain English

- Reads incoming events (cars arriving, buttons pressed, emergencies).

- Decides when the current phase has run long enough.

- Chooses the next phase, respecting the safety invariant.

- Sends commands to the four traffic lights and waits for their acknowledgements.

- Writes the new phase into shared memory so the GUI and logger see it.

## 5.2 Initial setup

int main(int argc, char **argv) {

(void)argc; (void)argv;

/* attach to the IPC objects created by main.c */

int shm_id = ipc_open_shm();

int sem_id = ipc_open_sem();

int cmd_q  = ipc_open_q(KEY_CMD);

int evt_q  = ipc_open_q(KEY_EVT);

int log_q  = ipc_open_q(KEY_LOG);

shared_state_t *S = shmat(shm_id, NULL, 0);

/* ... main loop ... */

}

Note: the controller does NOT create the IPC objects. main.c does. The controller only attaches to existing ones. This is the single-creator pattern: it removes the need for any "is it already created?" logic in nine different places.

## 5.3 Draining the event queue

static void drain_events(int evt_q, shared_state_t *S, int sem_id) {

evt_msg_t m;

while (msgrcv(evt_q, &m, sizeof(m)-sizeof(long), 0, IPC_NOWAIT) > 0) {

sem_lock(sem_id);

switch (m.kind) {

case EVT_VEHICLE:

if (S->vehicle_queue[m.dir] == 0)

S->first_arrival[m.dir] = m.when;

S->vehicle_queue[m.dir]++;

break;

case EVT_PEDESTRIAN:

if (!S->ped_request[m.dir]) {

S->ped_request[m.dir] = 1;

S->ped_request_t[m.dir] = m.when;

}

break;

case EVT_EMERGENCY:

S->emergency_active = 1;

S->emergency_dir = m.dir;

S->emergency_start = m.when;

break;

}

sem_unlock(sem_id);

}

}

- msgrcv with mtype=0 means "give me any message." IPC_NOWAIT means "return -1 with errno=ENOMSG if there is nothing."

- We loop until the queue is empty so a burst of arrivals is processed in one batch, not one per tick.

- first_arrival[dir] is set only on the transition from zero to non-zero queue, so it tracks the oldest still-waiting car.

- A pedestrian button stays "pressed" until the corresponding phase actually happens; pressing twice does not double-count.

## 5.4 The adaptive green duration

This is the cleverest formula in the project. The base case is to make every green phase T_GREEN_MIN seconds. But if one direction has more queued cars than the other, we extend the green for the busier side:

static int adaptive_green(const shared_state_t *S, int active_dir1, int active_dir2,

int cross_dir1, int cross_dir2) {

int q_active = S->vehicle_queue[active_dir1] + S->vehicle_queue[active_dir2];

int q_cross  = S->vehicle_queue[cross_dir1]  + S->vehicle_queue[cross_dir2];

int t = S->cfg.t_green_min + q_active - q_cross/2;

if (t < S->cfg.t_green_min) t = S->cfg.t_green_min;

if (t > S->cfg.t_green_max) t = S->cfg.t_green_max;

return t;

}

If NS has 5 cars and EW has 0, the active phase gets T_GREEN_MIN + 5 - 0/2 = MIN+5 seconds, clamped to MAX. The cross-direction queue is halved so the formula is biased toward extending the active phase, but a very long queue on the other side still pulls the duration down toward MIN — at which point the cycle flips and the other side gets its turn.

Why: We deliberately do not extend indefinitely. T_GREEN_MAX is a hard ceiling so the cross-direction never starves. This is a real-time deadline: every direction is guaranteed a green within at most T_GREEN_MAX + T_YELLOW + T_ALL_RED + T_GREEN_MAX + T_YELLOW + T_ALL_RED seconds.

## 5.5 The main loop

while (!S->shutdown) {

drain_events(evt_q, S, sem_id);

sem_lock(sem_id);

time_t now = time(NULL);

int elapsed = (int)(now - S->phase_start);

/* 1. emergency preemption */

if (S->emergency_active && S->phase != PHASE_EMERGENCY_NS

&& S->phase != PHASE_EMERGENCY_EW) {

handle_emergency(...);

}

/* 2. pedestrian opening */

else if (need_pedestrian(S) && S->phase == PHASE_ALL_RED) {

enter_phase(S, PHASE_PEDESTRIAN, ...);

}

/* 3. normal cycle */

else if (elapsed >= S->phase_duration) {

advance_phase(S, cmd_q, log_q);

}

/* 4. integrity check */

if (!state_is_safe(S))

send_log(log_q, 2, "SAFETY VIOLATION in phase %d", S->phase);

sem_unlock(sem_id);

usleep(50000);   /* 20 Hz */

}

The order matters: emergency wins over pedestrian, pedestrian wins over the normal cycle. Then we sanity-check after every transition. The safety check should never fire in correct code; if it does, the log entry is the alarm bell.

## 5.6 Phase transition: enter_phase

static void enter_phase(shared_state_t *S, phase_t next, int duration,

int cmd_q, int log_q) {

/* compute the target color for each light */

light_color_t target[NUM_DIRS];

color_table_for_phase(next, target);

/* send a command per light that needs to change */

for (int d = 0; d < NUM_DIRS; d++) {

if (S->lights[d] != target[d]) {

cmd_msg_t m = { .mtype = MTYPE_CMD_BASE + d,

.dir = d, .target = target[d], .when = time(NULL) };

msgsnd(cmd_q, &m, sizeof(m)-sizeof(long), 0);

}

}

/* wait for one ACK per command, with a 1-s budget */

wait_for_acks(cmd_q, target, S);

S->phase = next;

S->phase_start = time(NULL);

S->phase_duration = duration;

send_log(log_q, 0, "entered phase %s, duration %d s",

PHASE_NAMES[next], duration);

}

The interesting detail: we only send a command to a light if it actually needs to change. If a light is already RED and the next phase wants it RED, we skip it. Less work, less log noise, and lights never receive nonsense commands.

Gotcha: msgsnd is called WITHOUT IPC_NOWAIT here. The CMD queue is small (the kernel default is 8 KB) so a blocking send is fine; the lights drain commands immediately. If we used NOWAIT and the queue filled, we would silently drop a transition — a safety violation.

## 5.7 Emergency handling, step by step

When emergency_active is set, the controller takes the following sequence:

- Decide whether the emergency direction is N/S (becomes EMERGENCY_NS) or E/W (EMERGENCY_EW).

- If the current phase has any GREEN, first send commands to set those GREENs to YELLOW. Hold for T_YELLOW seconds.

- Send commands to set all conflicting lights to RED. Hold for T_ALL_RED seconds.

- Send commands to set the emergency direction to GREEN. Hold until the emergency clears (modelled as T_EMERGENCY_RESPONSE seconds in this simulation).

- Clear emergency_active, send all lights back to RED, then resume the normal cycle from PHASE_ALL_RED.

Why: We never go straight from a green of the cross direction to a green of the emergency direction. That would violate yellow-before-red and could surprise a driver. The cost is up to T_YELLOW + T_ALL_RED seconds of additional response time, which is well within the spec's T_EMERGENCY_RESPONSE budget.

# Chapter 6 — traffic_light.c

Four instances of this binary run, one per direction. Each instance is invoked by main.c with an integer argument 0..3 that tells it which direction it represents. Otherwise the code is identical.

## 6.1 The blocking receive

int dir = atoi(argv[1]);

long my_mtype = MTYPE_CMD_BASE + dir;

while (!S->shutdown) {

cmd_msg_t m;

/* block on our own dedicated mtype */

if (msgrcv(cmd_q, &m, sizeof(m)-sizeof(long), my_mtype, 0) <= 0)

continue;  /* EINTR on signal */

apply_command(S, sem_id, log_q, &m);

send_ack(cmd_q, &m);

}

Each light uses msgrcv WITHOUT IPC_NOWAIT, so the call blocks until a message arrives. While blocked the process consumes zero CPU — the kernel parks it on the queue's wait list. This is dramatically more efficient than polling and is exactly the textbook use of message queues.

Why: Filtering by mtype at the kernel level is essential. If we used mtype=0 (any message), each light would have to receive every other light's commands and throw away three quarters of them — adding contention and complexity.

## 6.2 Applying the command

static void apply_command(shared_state_t *S, int sem_id, int log_q,

const cmd_msg_t *m) {

/* fault injection: 0.2% chance of a stuck-light fault */

int faulty = (rand() % 1000) < 2;

sem_lock(sem_id);

if (faulty) {

S->lights[m->dir] = LIGHT_RED;   /* fail-safe to RED */

S->light_faulty[m->dir] = 1;

send_log(log_q, 2, "FAULT: light %s stuck, forcing RED",

DIR_NAMES[m->dir]);

} else {

S->lights[m->dir] = m->target;

S->light_change_t[m->dir] = m->when;

S->light_faulty[m->dir] = 0;

send_log(log_q, 0, "light %s -> %s",

DIR_NAMES[m->dir], COLOR_NAMES[m->target]);

}

sem_unlock(sem_id);

usleep(20000);  /* 20 ms physical actuation time */

}

Three things to notice:

- Fail-safe to RED. A real traffic light, when it loses confidence in its own state, defaults to flashing red. Our simulated fault does the same — RED is always safe.

- The 20 ms sleep models the small physical actuation time of a relay. It is small enough not to affect timing visibly, but big enough that the GUI can show the brief transition.

- Fault probability is 0.2 %. Over an hour of running you will see a handful of faults log themselves. Lower this to 0 if you do not want it during a demo.

## 6.3 Acknowledging

static void send_ack(int cmd_q, const cmd_msg_t *m) {

ack_msg_t a = {

.mtype = MTYPE_ACK,

.dir = m->dir,

.reached = m->target,

.ok = 1,

};

msgsnd(cmd_q, &a, sizeof(a)-sizeof(long), 0);

}

The ACK shares the same queue as the command, just with a different mtype. The controller does msgrcv(..., MTYPE_ACK, ...) to pick up only acks. This keeps the number of message queues to three.

# Chapter 7 — Sensors: vehicle_detector, pedestrian, emergency

Three small generators that turn random numbers into events on the EVT queue. They could have been one process, but separating them makes the spawn rates independently tunable and makes it easy to replace one with a real sensor later.

## 7.1 vehicle_detector.c

while (!S->shutdown) {

for (int d = 0; d < NUM_DIRS; d++) {

if ((rand() % 100) < S->cfg.vehicle_spawn_rate) {

evt_msg_t m = {

.mtype = MTYPE_EVT_VEHICLE,

.dir = d,

.kind = EVT_VEHICLE,

.when = time(NULL),

};

msgsnd(evt_q, &m, sizeof(m)-sizeof(long), IPC_NOWAIT);

send_log(log_q, 0, "vehicle arrived from %s", DIR_NAMES[d]);

}

}

sleep(1);

}

- Once per second, roll the dice for each direction. At 25% rate you expect about one arrival per second total.

- Sending uses IPC_NOWAIT: if the controller is somehow backed up and the queue is full, we drop the arrival rather than freeze. A real intersection survives one missed car.

- Logging is at INFO severity — these are normal events, not errors.

## 7.2 pedestrian.c

Almost identical, but it loops once per 1.5 seconds and uses pedestrian_spawn_rate. Pedestrian button presses are slower than car arrivals.

## 7.3 emergency.c

while (!S->shutdown) {

if ((rand() % 1000) < S->cfg.emergency_spawn_rate) {

direction_t d = (direction_t)(rand() % NUM_DIRS);

evt_msg_t m = {

.mtype = MTYPE_EVT_EMERGENCY,

.dir = d,

.kind = EVT_EMERGENCY,

.when = time(NULL),

};

msgsnd(evt_q, &m, sizeof(m)-sizeof(long), 0);

send_log(log_q, 1, "EMERGENCY from %s", DIR_NAMES[d]);

sleep(20);    /* one emergency, then a cooldown */

}

sleep(2);

}

Emergencies are at WARN severity in the log so they are easy to grep for. After firing one, the process sleeps 20 seconds — without that cooldown you could get back-to-back emergencies that never let the normal cycle recover. The % 1000 denominator matches the per-mille meaning of the config key.

# Chapter 8 — logger.c

The logger is dead simple but does the most underappreciated job: it gets the disk write off the critical path of every other process.

## 8.1 The whole loop

FILE *f = fopen("logs/system.log", "a");

while (!S->shutdown) {

log_msg_t m;

if (msgrcv(log_q, &m, sizeof(m)-sizeof(long), 0, 0) <= 0)

continue;

time_t now = time(NULL);

struct tm tm; localtime_r(&now, &tm);

const char *sev = (m.severity == 2) ? "ERROR"

: (m.severity == 1) ? "WARN " : "INFO ";

fprintf(f, "%02d:%02d:%02d  %s  %s\n",

tm.tm_hour, tm.tm_min, tm.tm_sec, sev, m.text);

fflush(f);

}

fclose(f);

- Blocking msgrcv with mtype=0 — like a traffic light, the logger sleeps when there is nothing to do.

- fflush after every write so the log is up-to-date even if the system crashes. The cost is one extra syscall per log entry, which is well under the rate we generate.

- localtime_r is the thread-safe form of localtime. We do not use threads in this project, but the habit is worth keeping.

- Severity tags are padded to five characters so columns line up when you tail -f the file.

Why: Why not let each process write to the log file directly? Two reasons: (1) interleaved writes from nine processes can tear without expensive flock() locking, and (2) the disk syscall would happen inside whoever has the semaphore at that moment, slowing the whole pipeline.

# Chapter 9 — main.c: the launcher

main.c does four things, in order: parse the config, create the IPC objects, fork the nine children, and supervise them until shutdown.

## 9.1 The flow

int main(int argc, char **argv) {

if (argc < 2) { fprintf(stderr, "usage: %s config.cfg [--no-gui]\n", argv[0]); exit(1); }

int no_gui = (argc > 2 && !strcmp(argv[2], "--no-gui"));

config_t cfg; config_load(&cfg, argv[1]);

int shm_id = ipc_create_shm();

int sem_id = ipc_create_sem();

int cmd_q  = ipc_create_q(KEY_CMD);

int evt_q  = ipc_create_q(KEY_EVT);

int log_q  = ipc_create_q(KEY_LOG);

shared_state_t *S = shmat(shm_id, NULL, 0);

init_shared_state(S, &cfg);

/* install SIGTERM/SIGINT handlers that set S->shutdown */

signal(SIGINT,  on_signal);

signal(SIGTERM, on_signal);

spawn("./logger");

spawn("./controller");

for (int d = 0; d < NUM_DIRS; d++) {

char arg[2] = { '0' + d, 0 };

spawn_arg("./traffic_light", arg);

}

spawn("./vehicle_detector");

spawn("./pedestrian");

spawn("./emergency");

if (!no_gui) spawn("./gui_opengl");

/* supervise */

while (!S->shutdown) {

int status; pid_t p = waitpid(-1, &status, WNOHANG);

if (p > 0) handle_child_exit(p, status);

sleep(1);

}

/* graceful shutdown: signal every child, then ipc_destroy_all */

}

- Logger is spawned FIRST. Every other process writes to the log queue from the moment it starts, so the logger must be ready to drain.

- Controller is spawned BEFORE the sensors so the controller is already pumping the EVT queue when events start arriving.

- The four traffic lights each get a single-character argument for their direction index.

- The GUI is the last thing spawned and may be skipped with --no-gui for headless runs.

## 9.2 The supervisor

static void handle_child_exit(pid_t pid, int status) {

const char *name = name_of(pid);

if (WIFEXITED(status))

fprintf(stderr, "child %s exited normally (code %d)\n", name, WEXITSTATUS(status));

else

fprintf(stderr, "child %s killed by signal %d\n", name, WTERMSIG(status));

/* in a production system we would respawn; for the project we shut down. */

shutdown_all();

}

If any child dies unexpectedly, we tear down the whole system. This is safer than respawning during a demo because a respawn might mask a real bug.

## 9.3 Cleanup

static void ipc_destroy_all(int shm_id, int sem_id,

int cmd_q, int evt_q, int log_q) {

shmdt(S);

shmctl(shm_id, IPC_RMID, NULL);

semctl(sem_id, 0, IPC_RMID);

msgctl(cmd_q, IPC_RMID, NULL);

msgctl(evt_q, IPC_RMID, NULL);

msgctl(log_q, IPC_RMID, NULL);

}

- shmdt detaches our view of the shared segment.

- IPC_RMID actually destroys the kernel object. Without this, the IPC objects survive the program (you can see them with ipcs).

- If main is SIGKILLed before reaching this, the IPC objects are leaked. Run "make ipcclean" or "ipcrm" to remove them by hand.

# Chapter 10 — gui_opengl.c: visualization

The GUI is fundamentally a read-only observer of shared memory. It never writes to the blackboard except to push synthetic events through the EVT queue when the user presses a key. Treat it as a separate display application that happens to be in the same process tree.

## 10.1 Snapshot pattern

Every frame, the GUI does this:

static void render_frame(void) {

shared_state_t snap;

sem_lock(sem_id);

snap = *S;          /* copy everything */

sem_unlock(sem_id);

/* render from snap, NOT from S */

}

Why: OpenGL drawing can take 10–30 ms. Holding the semaphore that long would freeze the controller and the lights. By taking a fast snapshot under the lock and rendering from the snapshot, the GUI holds the lock for only a few microseconds.

## 10.2 The 3-D scene, layer by layer

- A green ground plane covering everything.

- Gray asphalt: two perpendicular roads forming a cross at the origin, with yellow dashed lane dividers.

- White zebra crossings on each of the four arms, just before the intersection.

- White stop lines at the edge of the intersection itself.

- Four corner poles, each with a vertical 3-lamp traffic light. Lamps glow via emissive material when active.

- Cars: composed of a body box, a roof box, and four spheres for wheels. Color from a 6-element palette indexed by the per-direction car index.

- An emergency vehicle (when active) drawn in white with a flashing red/blue light-bar built from two small boxes that pulse out of phase.

- Stick-figure pedestrians animated across the zebra crossing during PHASE_PEDESTRIAN.

- A translucent HUD quad overlaid in screen space at the top of the window, showing: phase name, countdown to next transition, per-direction queue lengths, emergency flag, and the keyboard help line.

## 10.3 Materials and lighting

static void set_lamp_material(int on, light_color_t color) {

GLfloat off_diff[4]  = {0.15f, 0.15f, 0.15f, 1.0f};

GLfloat off_emit[4]  = {0.0f, 0.0f, 0.0f, 1.0f};

GLfloat r_diff[4]    = {0.9f, 0.1f, 0.1f, 1.0f};

GLfloat r_emit[4]    = {1.0f, 0.2f, 0.2f, 1.0f};

/* ... yellow and green similarly ... */

if (!on) {

glMaterialfv(GL_FRONT, GL_DIFFUSE,  off_diff);

glMaterialfv(GL_FRONT, GL_EMISSION, off_emit);

} else if (color == LIGHT_RED) {

glMaterialfv(GL_FRONT, GL_DIFFUSE,  r_diff);

glMaterialfv(GL_FRONT, GL_EMISSION, r_emit);

}

/* etc. */

}

The trick that makes a green light look like it is glowing is GL_EMISSION, which adds color regardless of the scene's lighting. The other two lamps in the same pole have zero emission and a dim diffuse color so they look dark even when sunlit.

## 10.4 Camera control

- Arrow keys rotate the camera around the intersection.

- + and - zoom in and out by adjusting the gluLookAt eye distance.

- The view is gluPerspective with a 50° field of view — wider than a single car driver would see, narrower than a fisheye.

- h toggles the HUD overlay. Useful for screenshots.

- q quits the GUI cleanly (signals main to shut everything down).

## 10.5 User-injected events

static void keyboard(unsigned char key, int x, int y) {

switch (key) {

case 'n': case 'N': inject_event(EVT_VEHICLE,    DIR_N); break;

case 's': case 'S': inject_event(EVT_VEHICLE,    DIR_S); break;

case 'e': case 'E': inject_event(EVT_VEHICLE,    DIR_E); break;

case 'w': case 'W': inject_event(EVT_VEHICLE,    DIR_W); break;

case '1': inject_event(EVT_EMERGENCY, DIR_N); break;

case '2': inject_event(EVT_EMERGENCY, DIR_S); break;

case '3': inject_event(EVT_EMERGENCY, DIR_E); break;

case '4': inject_event(EVT_EMERGENCY, DIR_W); break;

case 'p': inject_event(EVT_PEDESTRIAN, DIR_N); break;

case 'q': quit_all(); break;

}

}

The keyboard handler pushes events into exactly the same EVT queue that the random sensors use. The controller cannot tell whether an event came from a random number generator or a key press — which is the right design: there should be no privileged source of events.

Tip: During the demo, press '1' to fire an emergency from the North. The lights you see green will go yellow within seconds, then red, then North will get a green — exactly the sequence required by the spec.

# Chapter 11 — Safety, Liveness, and What Could Go Wrong

## 11.1 The safety invariants we enforce

- No conflicting greens. Checked by state_is_safe() after every transition; checked structurally by the phase enum (no phase exists in which both NS and EW have non-red lights).

- Yellow before red. Enforced by the phase machine: NS_YELLOW always precedes ALL_RED on the way out of NS_GREEN. Even emergency preemption goes through yellow.

- Pedestrians only when crossing direction is red. PEDESTRIAN phase is reachable only from ALL_RED, when every light is already red.

- Emergency cannot create unsafe transitions. handle_emergency() emits the same yellow→red→green sequence even under time pressure.

## 11.2 Liveness — nothing starves

- Each direction sees a green at least every (T_GREEN_MAX + T_YELLOW + T_ALL_RED) × 2 seconds. With defaults that is at most 40 s.

- Pedestrians wait at most T_PEDESTRIAN_MAX_WAIT seconds (45 s default) before the controller forces a walk phase even with no organic ALL_RED.

- Logger: never blocks because every send uses IPC_NOWAIT.

- Controller: never blocks indefinitely because every wait_for_ack has a timeout.

## 11.3 Where deadlock could have happened, and how we avoided it

- Two-lock ordering. We have one semaphore. There is no second lock to deadlock against.

- Holding the lock across blocking syscalls. We never hold the lock during msgrcv or any blocking call — the controller releases the lock before usleep, the lights release before their next msgrcv.

- Process crash with lock held. Every semop has SEM_UNDO, so the kernel undoes our hold automatically if we die.

- Message-queue full. The log queue uses IPC_NOWAIT. The CMD queue is naturally drained instantly by four blocked lights. The EVT queue is drained on every controller tick.

## 11.4 Failure modes we tolerate

| Fault | Outcome |
| --- | --- |
| A traffic light's stuck-relay fault | Light snaps to RED, logs ERROR, controller sees it next tick |
| GUI killed | Rest of system keeps running; main detects exit and shuts down |
| Sensor killed | No new events of that kind; controller keeps cycling on existing state |
| Logger killed | LOG queue fills, IPC_NOWAIT drops new entries; system stays safe but loses observability |
| main killed with SIGKILL | IPC objects leaked; recover with 'make ipcclean' |

# Chapter 12 — Quick Reference

## 12.1 Key compile-time constants

| Constant | Value | File |
| --- | --- | --- |
| NUM_DIRS | 4 | common.h |
| MTYPE_CMD_BASE | 100 | common.h |
| MTYPE_ACK | 200 | common.h |
| MTYPE_EVT_VEHICLE | 10 | common.h |
| MTYPE_EVT_PEDESTRIAN | 11 | common.h |
| MTYPE_EVT_EMERGENCY | 12 | common.h |
| KEY_SHM, KEY_SEM, KEY_CMD, KEY_EVT, KEY_LOG | ftok-derived | ipc_init.h |
| Controller tick | 50 000 µs (20 Hz) | controller.c |
| Sensor tick | 1 000 000 µs (1 Hz) | vehicle_detector.c |

## 12.2 Keyboard cheat-sheet (GUI)

| Key | Effect |
| --- | --- |
| n / s / e / w | Inject a vehicle arrival from that direction |
| N / S / E / W (shift) | Inject a pedestrian request from that direction |
| 1 / 2 / 3 / 4 | Inject an emergency from N / S / E / W |
| Arrow keys | Rotate camera |
| + / - | Zoom in / out |
| h | Toggle HUD |
| q | Quit (shuts down whole system) |

## 12.3 Files to grep when something looks wrong

| Symptom | Grep in |
| --- | --- |
| "SAFETY VIOLATION" in log | controller.c (check_safety, advance_phase) |
| Lights freeze for >5 seconds | controller.c main loop, traffic_light.c msgrcv |
| IPC errors at startup | ipc_init.h (ftok target file must exist), main.c spawn order |
| GUI black / blank | gui_opengl.c init_gl, materials, camera Z too small |
| No log entries | logger.c fopen path, send_log calls |
| Stale IPC after crash | Run 'make ipcclean' or 'ipcrm -a' |

## 12.4 Useful one-liners during testing

# watch the live log

tail -f logs/system.log

# only show errors/warnings

tail -f logs/system.log | grep -E 'WARN|ERROR'

# see if any IPC objects survived a crash

ipcs

# remove all IPC objects for the current user

ipcrm -a

# rebuild with debugging symbols and run under gdb

make debug && gdb --args ./main config/system.cfg --no-gui

## 12.5 Answers to likely demo questions

### Q: Why processes instead of threads?

Fault isolation, true preemption by the Linux scheduler, and direct alignment with the spec's IPC requirement. A thread-based system needs only a mutex; an IPC-based system actually exercises shm + sem + msq.

### Q: Why one semaphore, not many?

Because we only have one shared region — the blackboard — and one semaphore is sufficient to protect it. Adding more semaphores would invite lock-ordering bugs without gaining concurrency, since reads are short.

### Q: Why message queues for some signals and shared memory for state?

Message queues are best for asynchronous events that have a sender and a receiver — commands, acks, sensor readings. Shared memory is best for state that everyone reads but few people write — current lights, queues, phase. Using each tool for what it is best at keeps the code natural.

### Q: How do you prevent two conflicting greens?

Three layers: (1) the phase enum has no phase in which conflicting directions are simultaneously non-red; (2) every transition goes through yellow then ALL_RED before another green; (3) state_is_safe() is checked after every transition and logs an ERROR if violated.

### Q: What happens on shutdown?

SIGINT or SIGTERM sets S->shutdown. Each child sees this and exits its main loop. main waitpids on each, then calls ipc_destroy_all to remove the shared memory, semaphore, and three message queues from the kernel.

### Q: Why is the GUI optional?

So the grader can SSH in headlessly and still run the full simulation. The GUI does not affect correctness; it is purely an observer.